实验4：流水线设计

具体功能：

1）32位MIPS架构的cpu5级流水线，实现13条指令

2）对数据冲突采取插入空操作（nops）、重定向（forward）及阻塞（stall）方法，对控制冲突采取提前预测分支（eqcmp），如果判断将因分支前置产生数据冲突，即同样使用插入空操作、重定向及阻塞方法解决。

3）实验板上的开关用于模拟输入操作数和其它控制输入。

4）实验板上的8位数码管输出运算结果。

【一】流水线CPU与仿真

1.	实验方案

流水线CPU将每条指令的执行分成5个阶段，采用时间并行的方式，进行取指令——译码——执行——存储器——写回的过程，每个阶段的时间长度以最慢阶段的为准，抽象表示如下：（为了区分寄存器的写/读状态，规定寄存器在下降沿前半个周期写入，在上升沿后半个周期读取）

相较单周期CPU，流水线CPU在每个阶段间增添了暂存寄存器保存各控制信号的中间状态，以此并行执行。指令执行框图如下：

流水线CPU用于阶段间存在复用，存在数据冲突及控制冲突，前者指寄存器值还未写回寄存器文件在下一阶段已经开始使用，后者指在取下一条指令时，分支跳转尚未确定。

为了解决由于指令依赖于另一条还未结束指令的结果，对数据冲突采取插入空操作（nops）、重定向（forward）及阻塞（stall）方法，对控制冲突采取提前预测分支（eqcmp），如果判断将因分支前置产生数据冲突，即同样使用插入空操作、重定向及阻塞方法解决。

分析冲突解决具体的执行方法：

- 插入空操作（nops）

插入空指令以使提前调用的指令后移。

- 重定向（forward）

当执行阶段（E）中的指令有一个源寄存器rsE、rtE与存储器阶段（M）或写回阶段（W）中的目的寄存器writeRegM、writeRegW相匹配时，即将M或W阶段应存入寄存器中的值提前定向到下一指令的E阶段。

- 阻塞（stall）

当lw的目的寄存器rtE与译码阶段（D）中的源操作数rsD、rtD相匹配时，由于需要获取的值还未出现，故无法使用重定向进行提前获取，需要进行阻塞。

使用信号StallF、StallD迫使取指阶段（F）、译码阶段（D）流水寄存器保持原来的值，使用信号FlushE清除执行阶段（E）流水线寄存器的内容，并去掉无效控制信号，以防止更新内存和状态。

- 分支预测（eqcmp）

采用专门的相等比较器，在译码阶段结束时确定下一个PC，进行分支预测。如果分支需要跳转，则仅下一指令完成第一阶段，只需要抛弃下一指令即可。

由此，处理所有冲突的流水线CPU结构框图如下：

2. 关键代码

主要呈现流水线CPU执行流水指令的部分

- 控制器controller

module controller(

input  logic       clk, reset,

input  logic [5:0] opD, functD,

input  logic       flushE, equalD,

output logic       memtoregE, memtoregM, memtoregW, memwriteM,

output logic       pcsrcD, branchD, alusrcE,

output logic       regdstE, regwriteE, regwriteM, regwriteW,

output logic       jumpD,

output logic [2:0] alucontrolE);

logic [1:0] aluopD;

logic       memtoregD, memwriteD, alusrcD, regdstD, regwriteD;

logic [2:0] alucontrolD;

logic       memwriteE;

maindec md(opD, memtoregD, memwriteD, branchD,

alusrcD, regdstD, regwriteD, jumpD, aluopD);

aludec ad(functD, aluopD, alucontrolD);

assign pcsrcD = branchD & equalD;

floprc #(8) regE(clk, reset, flushE,

{memtoregD, memwriteD, alusrcD, regdstD, regwriteD, alucontrolD},

{memtoregE, memwriteE, alusrcE, regdstE, regwriteE, alucontrolE});

flopr  #(3) regM(clk, reset,

{memtoregE, memwriteE, regwriteE},

{memtoregM, memwriteM, regwriteM});

flopr  #(2) regW(clk, reset,

{memtoregM, regwriteM},

{memtoregW, regwriteW});

Endmodule

- 数据通路datapath

module datapath(

input  logic        clk, reset,

input  logic        memtoregE, memtoregM, memtoregW,

input  logic        pcsrcD, branchD,

input  logic        alusrcE, regdstE,

input  logic        regwriteE, regwriteM, regwriteW,

input  logic        jumpD,

input  logic [2:0]  alucontrolE,

output logic        equalD,

output logic [31:0] pcF,

input  logic [31:0] instrF,

output logic [31:0] aluoutM, writedataM,

input  logic [31:0] readdataM,

output logic [5:0]  opD, functD,

output logic        flushE);

logic        forwardaD, forwardbD;

logic [1:0]  forwardaE, forwardbE;

logic        stallF, stallD;

logic [4:0]  rsD, rtD, rdD, rsE, rtE, rdE;

logic [4:0]  writeregE, writeregM, writeregW;

logic        flushD;

logic [31:0] pcnextFD, pcnextbrFD, pcplus4F, pcbranchD;

logic [31:0] signimmD, signimmE, signimmshD;

logic [31:0] srcaD, srca2D, srcaE, srca2E;

logic [31:0] srcbD, srcb2D, srcbE, srcb2E, srcb3E;

logic [31:0] pcplus4D, instrD;

logic [31:0] aluoutE, aluoutW;

logic [31:0] readdataW, resultW;

// hazard detection

hazard h(rsD, rtD, rsE, rtE, writeregE, writeregM, writeregW,

regwriteE, regwriteM, regwriteW,

memtoregE, memtoregM, branchD,

forwardaD, forwardbD, forwardaE, forwardbE,

stallF, stallD, flushE);

// next PC logic (operates in fetch and decode)

mux2 #(32) pcbrmux(pcplus4F, pcbranchD, pcsrcD, pcnextbrFD);

mux2 #(32) pcmux(pcnextbrFD, {pcplus4D[31:28], instrD[25:0], 2'b00},

jumpD, pcnextFD);

// register file (operates in decode and writeback)

regfile rf(clk, regwriteW, rsD, rtD, writeregW,

resultW, srcaD, srcbD);

// Fetch

flopenr #(32) pcreg(clk, reset, ~stallF, pcnextFD, pcF);

adder pcadd1(pcF, 32'b100, pcplus4F);

// Decode

flopenr  #(32) r1D(clk, reset, ~stallD, pcplus4F, pcplus4D);

flopenrc #(32) r2D(clk, reset, ~stallD, flushD, instrF, instrD);

signext se(instrD[15:0], signimmD);

sl2 immsh(signimmD, signimmshD);

adder pcadd2(pcplus4D, signimmshD, pcbranchD);

mux2 #(32) forwardadmux(srcaD, aluoutM, forwardaD, srca2D);

mux2 #(32) forwardbdmux(srcbD, aluoutM, forwardbD, srcb2D);

eqcmp comp(srca2D, srcb2D, equalD);

assign opD    = instrD[31:26];

assign functD = instrD[5:0];

assign rsD    = instrD[25:21];

assign rtD    = instrD[20:16];

assign rdD    = instrD[15:11];

assign flushD = pcsrcD | jumpD;

// Execute

floprc #(32) r1E(clk, reset, flushE, srcaD, srcaE);

floprc #(32) r2E(clk, reset, flushE, srcbD, srcbE);

floprc #(32) r3E(clk, reset, flushE, signimmD, signimmE);

floprc #(5)  r4E(clk, reset, flushE, rsD, rsE);

floprc #(5)  r5E(clk, reset, flushE, rtD, rtE);

floprc #(5)  r6E(clk, reset, flushE, rdD, rdE);

mux3 #(32) forwardaemux(srcaE, resultW, aluoutM, forwardaE, srca2E);

mux3 #(32) forwardbemux(srcbE, resultW, aluoutM, forwardbE, srcb2E);

mux2 #(32) srcbmux(srcb2E, signimmE, alusrcE, srcb3E);

alu alu(srca2E, srcb3E, alucontrolE, aluoutE);

mux2 #(5)  wrmux(rtE, rdE, regdstE, writeregE);

// Memory

flopr #(32) r1M(clk, reset, srcb2E, writedataM);

flopr #(32) r2M(clk, reset, aluoutE, aluoutM);

flopr #(5)  r3M(clk, reset, writeregE, writeregM);

// Writeback

flopr #(32) r1W(clk, reset, aluoutM, aluoutW);

flopr #(32) r2W(clk, reset, readdataM, readdataW);

flopr #(5)  r3W(clk, reset, writeregM, writeregW);

mux2  #(32) resmux(aluoutW, readdataW, memtoregW, resultW);

endmodule

- 冲突处理hazard

module hazard(

input  logic [4:0] rsD, rtD, rsE, rtE,

input  logic [4:0] writeregE, writeregM, writeregW,

input  logic       regwriteE, regwriteM, regwriteW,

input  logic       memtoregE, memtoregM, branchD,

output logic       forwardaD, forwardbD,

output logic [1:0] forwardaE, forwardbE,

output logic       stallF, stallD, flushE);

logic lwstallD, branchstallD;

assign forwardaD = (rsD != 0 & rsD == writeregM & regwriteM);

assign forwardbD = (rtD != 0 & rtD == writeregM & regwriteM);

always_comb

begin

forwardaE = 2'b00; forwardbE = 2'b00;

if (rsE != 0)

if      (rsE == writeregM & regwriteM) forwardaE = 2'b10;

else if (rsE == writeregW & regwriteW) forwardaE = 2'b01;

if (rtE != 0)

if      (rtE == writeregM & regwriteM) forwardbE = 2'b10;

else if (rtE == writeregW & regwriteW) forwardbE = 2'b01;

end

assign lwstallD = memtoregE & (rtE == rsD | rtE == rtD);

assign branchstallD = branchD &

(regwriteE & (writeregE == rsD | writeregE == rtD) |

memtoregM & (writeregM == rsD | writeregM == rtD));

assign stallD = lwstallD | branchstallD;

assign stallF = stallD;

assign flushE = stallD;

endmodule

3.仿真结果：

【二】流水线CPU_add：仿真

1.	实验方案

在原有指令的基础上添加andi、ori、bne指令，修改后的模块框图与单周期CPU类似：

对比增添指令前后的模块框图，发现主要改动在于：1）控制单元信号位数扩展，主译码器增添ImmExt，BranchBne两位，ALU译码器的控制信号ALUOp位数增加，增加分支指令内部判断逻辑；2）立即数扩展增加0扩展，复用器进行选择，当立即数被用于执行andi、ori指令时，需要进行0扩展；被用于执行addi指令时，需要进行符号扩展。

对于新增指令无需进行新的冲突处理，andi、ori可能产生的冲突为数据冲突，branchbne产生的分支控制冲突可沿用branch的冲突处理方法进行处理，只需在主译码器进行control译码时，将对应的branch位与branchbne位均置为1：

并在原有代码上将aluop由2位扩展为3位，alucontrol由15位扩展为18位，增加signzero零扩展模块，以及pcsrc复用器模块。

2.关键代码

显示更改部分：

- 控制器controller/maindec/aludec

module controller(

input  logic       clk, reset,

input  logic [5:0] opD, functD,

input  logic       flushE, equalD,

output logic       memtoregE, memtoregM, memtoregW, memwriteM,

output logic       pcsrcD, branchD, branchbneD, alusrcE,

output logic       regdstE, regwriteE, regwriteM, regwriteW,

output logic       jumpD, immext,

output logic [2:0] alucontrolE);

logic [2:0] aluopD;

logic       memtoregD, memwriteD, alusrcD, regdstD, regwriteD;

logic [2:0] alucontrolD;

logic       memwriteE;

maindec md(opD, memtoregD, memwriteD, branchD, branchbneD,

alusrcD, regdstD, regwriteD, jumpD, immext, aluopD);

aludec ad(functD, aluopD, alucontrolD);

assign pcsrcD = (branchD & equalD) | (branchbneD & (~equalD));

floprc #(8) regE(clk, reset, flushE,

{memtoregD, memwriteD, alusrcD, regdstD, regwriteD, alucontrolD},

{memtoregE, memwriteE, alusrcE, regdstE, regwriteE, alucontrolE});

flopr  #(3) regM(clk, reset,

{memtoregE, memwriteE, regwriteE},

{memtoregM, memwriteM, regwriteM});

flopr  #(2) regW(clk, reset,

{memtoregM, regwriteM},

{memtoregW, regwriteW});

Endmodule

module maindec(

input  logic [5:0] op,

output logic       memtoreg, memwrite,

output logic       branch, branchbne, alusrc,

output logic       regdst, regwrite,

output logic       jump, immext,

output logic [2:0] aluop);

logic [11:0] controls;

assign {regwrite, regdst, alusrc, branch, memwrite,

memtoreg, jump, aluop, immext, branchbne} = controls;

always_comb

case(op)

6'b000000: controls <= 12'b110000001000; //RTYPE

6'b100011: controls <= 12'b101001000000; //LW

6'b101011: controls <= 12'b001010000000; //SW

6'b000100: controls <= 12'b000100000100; //BEQ

6'b001000: controls <= 12'b101000000000; //ADDI

6'b000010: controls <= 12'b000000100000; //J

6'b000101: controls <= 12'b000100000101; //BNE:branch=1

6'b001101: controls <= 12'b101000001110; //ORI

6'b001100: controls <= 12'b101000010010; //ANDI

default:   controls <= 12'bxxxxxxxxxxx; //illegal op

endcase

endmodule

module aludec(

input  logic [5:0] funct,

input  logic [2:0] aluop,

output logic [2:0] alucontrol);

always_comb

case(aluop)

3'b000: alucontrol <= 3'b010; //add(for lw/sw/addi)

3'b001: alucontrol <= 3'b110; //sub(for beg/bne)

3'b100: alucontrol <= 3'b000;

3'b011: alucontrol <= 3'b001;

default: case(funct) //R-type instructions

6'b100000: alucontrol <= 3'b010; //add

6'b100010: alucontrol <= 3'b110; //sub

6'b100100: alucontrol <= 3'b000; //and

6'b100101: alucontrol <= 3'b001; //or

6'b101010: alucontrol <= 3'b111; //slt

default:   alucontrol <= 3'bxxx; //???

endcase

endcase

endmodule

- 数据通路datapath

module datapath(

input  logic        clk, reset,

input  logic        memtoregE, memtoregM, memtoregW,

input  logic        pcsrcD, branchD,

input  logic        alusrcE, regdstE,

input  logic        regwriteE, regwriteM, regwriteW,

input  logic        jumpD,

input  logic [2:0]  alucontrolE,

output logic        equalD,

output logic [31:0] pcF,

input  logic [31:0] instrF,

output logic [31:0] aluoutM, writedataM,

input  logic [31:0] readdataM,

output logic [5:0]  opD, functD,

output logic        flushE,

input  logic        immext);

logic        forwardaD, forwardbD;

logic [1:0]  forwardaE, forwardbE;

logic        stallF, stallD;

logic [4:0]  rsD, rtD, rdD, rsE, rtE, rdE;

logic [4:0]  writeregE, writeregM, writeregW;

logic        flushD;

logic [31:0] pcnextFD, pcnextbrFD, pcplus4F, pcbranchD;

logic [31:0] signimmD, signimmE, signimmshD;

logic [31:0] zeroimm, imm;

logic [31:0] srcaD, srca2D, srcaE, srca2E;

logic [31:0] srcbD, srcb2D, srcbE, srcb2E, srcb3E;

logic [31:0] pcplus4D, instrD;

logic [31:0] aluoutE, aluoutW;

logic [31:0] readdataW, resultW;

// hazard detection

hazard h(rsD, rtD, rsE, rtE, writeregE, writeregM, writeregW,

regwriteE, regwriteM, regwriteW,

memtoregE, memtoregM, branchD,

forwardaD, forwardbD, forwardaE, forwardbE,

stallF, stallD, flushE);

// next PC logic (operates in fetch and decode)

mux2 #(32) pcbrmux(pcplus4F, pcbranchD, pcsrcD, pcnextbrFD);

mux2 #(32) pcmux(pcnextbrFD, {pcplus4D[31:28], instrD[25:0], 2'b00},

jumpD, pcnextFD);

// register file (operates in decode and writeback)

regfile rf(clk, regwriteW, rsD, rtD, writeregW,

resultW, srcaD, srcbD);

// Fetch

flopenr #(32) pcreg(clk, reset, ~stallF, pcnextFD, pcF);

adder pcadd1(pcF, 32'b100, pcplus4F);

// Decode

flopenr  #(32) r1D(clk, reset, ~stallD, pcplus4F, pcplus4D);

flopenrc #(32) r2D(clk, reset, ~stallD, flushD, instrF, instrD);

signext se(instrD[15:0], signimmD);

ze ze(instrD[15:0], zeroimm);

sl2 immsh(signimmD, signimmshD);

adder pcadd2(pcplus4D, signimmshD, pcbranchD);

mux2 #(32) forwardadmux(srcaD, aluoutM, forwardaD, srca2D);

mux2 #(32) forwardbdmux(srcbD, aluoutM, forwardbD, srcb2D);

eqcmp comp(srca2D, srcb2D, equalD);

assign opD    = instrD[31:26];

assign functD = instrD[5:0];

assign rsD    = instrD[25:21];

assign rtD    = instrD[20:16];

assign rdD    = instrD[15:11];

assign flushD = pcsrcD | jumpD;

// Execute

floprc #(32) r1E(clk, reset, flushE, srcaD, srcaE);

floprc #(32) r2E(clk, reset, flushE, srcbD, srcbE);

floprc #(32) r3E(clk, reset, flushE, imm, signimmE);

floprc #(5)  r4E(clk, reset, flushE, rsD, rsE);

floprc #(5)  r5E(clk, reset, flushE, rtD, rtE);

floprc #(5)  r6E(clk, reset, flushE, rdD, rdE);

mux3 #(32) forwardaemux(srcaE, resultW, aluoutM, forwardaE, srca2E);

mux3 #(32) forwardbemux(srcbE, resultW, aluoutM, forwardbE, srcb2E);

mux2 #(32) extmux(signimmD, zeroimm, immext, imm);

mux2 #(32) srcbmux(srcb2E, signimmE, alusrcE, srcb3E);

alu alu(srca2E, srcb3E, alucontrolE, aluoutE);

mux2 #(5)  wrmux(rtE, rdE, regdstE, writeregE);

// Memory

flopr #(32) r1M(clk, reset, srcb2E, writedataM);

flopr #(32) r2M(clk, reset, aluoutE, aluoutM);

flopr #(5)  r3M(clk, reset, writeregE, writeregM);

// Writeback

flopr #(32) r1W(clk, reset, aluoutM, aluoutW);

flopr #(32) r2W(clk, reset, readdataM, readdataW);

flopr #(5)  r3W(clk, reset, writeregM, writeregW);

mux2 #(32)  resmux(aluoutW, readdataW, memtoregW, resultW);

endmodule

3. 仿真结果：

【三】流水线CPU_IO：仿真+上板

1.	实验方案

流水线CPU增添IO接口作为CPU与外设的连接桥梁，获得的模块框图与单周期类似：

发现：这里采用存储器映像IO接口，将IO接口的地址空间映像到存储器部分，使得CPU能够产生存储器读写信号和IO接口读写信号。IO接口一共分为两部分，一部分与CPU相连实现内部逻辑，一部分与外设相连控制输出。与外设相连的端口分为Switch高低端口、结果输出端口与状态端口。状态端口指当status[1]=1时，可输入新数据；当status[0]=1时，可输出新数据。

根据时序逻辑，用button和reset进行状态设置；根据组合逻辑，由加载的指令选择数据读入，并在数据存储译码器中进行复用器选择，由指令地址决定读取数据或者地址。

添加testbench代码进行仿真；添加TestIO.dat文件进行仿真与综合。

2.	关键代码

module top(

input logic CLK100MHZ,

input logic BTNC,//reset

input logic BTNL,//SW input data

input logic BTNR,//七段数码管显示

input logic [15:0] SW,//a:SW[15:8]，b:SW[7:0]

output logic [7:0] AN,

output logic [6:0] A2G,

output logic DP);

logic [31:0] pc;

logic [31:0]  instr;

imem imem(pc[7:2],instr); // output

logic Write;//写信号:可能是memWrite，也可能是ioWrite

logic [31:0] dataAdr, writeData, readData;

mips mips(CLK100MHZ,BTNC,pc,instr,Write,dataAdr,writeData,readData);

dMemoryDecoder dmd(.clk(CLK100MHZ),

.writeEN(Write),

.addr(dataAdr[7:0]),

.writeData(writeData),

.readData(readData), //output

.reset(BTNC),

.btnL(BTNL),

.btnR(BTNR),

.switch(SW),

.an(AN),    //output

.dp(DP),

.a2g(A2G)); //output

Endmodule

module dMemoryDecoder(

input  logic        clk,

input  logic        writeEN,

input  logic [7:0]  addr, //8bit

input  logic [31:0] writeData,

output logic [31:0] readData,

input  logic        reset,

input  logic        btnL,

input  logic        btnR,

input  logic [15:0] switch,

output logic [7:0]  an,

output logic        dp,

output logic [6:0]  a2g);

logic [11:0] led;

logic [31:0] digit;

assign digit = {switch,{4'b0000},led};

logic [31:0] ReadData1, ReadData2;

mux2 #(32)  rdmux(.d0(ReadData1),

.d1(ReadData2),

.s(addr[7]),

.y(readData));

dmem dMemory(.clk(clk),

.we(writeEN),

.a(addr),

.wd(writeData),

.rd(ReadData1));

io IO(.clk(clk),

.reset(reset),

.pRead(addr[7]),

.pWrite(writeEN),

.addr(addr[3:2]),

.pWriteData(writeData[11:0]),

.pReadData(ReadData2),

.buttonL(btnL),

.buttonR(btnR),

.switch(switch),

.led(led));

m7seg mux7seg(.x(digit),

.clk(clk),

.a2g(a2g),

.an(an),

.dp(dp));

Endmodule

module io(

//与CPU相连

input  logic        clk,

input  logic        reset,

input  logic        pRead,

input  logic        pWrite,

input  logic [1:0]  addr,

input  logic [11:0] pWriteData,

output logic [31:0] pReadData,

//与外设相连

input  logic        buttonL,

input  logic        buttonR,

input  logic [15:0] switch, //control

output logic [11:0] led); //digital

logic [1:0]  status;

logic [15:0] switchl;

logic [11:0] ledl;

always_ff @(posedge clk) begin

if(reset) begin

status <=  2'b00;

ledl   <=  12'h00;

switchl <= 16'h00;

end

else begin

//开关位置拨号，可以输入

if(buttonR) begin

status[1] <= 1;

switchl   <= switch;

end

//led显示准备好，可以输出

if(buttonL) begin

status[0] <= 1;

led       <= ledl;

end

//向数据输出端口输出（led）

if(pWrite & (addr==2'b01)) begin

ledl      <= pWriteData;

status[0] <= 0;

end

end

end

always_comb

if(pRead)

case(addr)

2'b11:   pReadData = {24'b0, switchl[15:8]};

2'b10:   pReadData = {24'b0, switchl[7:0]};

2'b00:   pReadData = {24'b0, 6'b0, status};

default: pReadData = 32'b0;

endcase

else

pReadData = 32'b0;

endmodule

module m7seg(

input logic [31:0] x,

input logic        clk,

output logic [6:0] a2g,

output logic [7:0] an,

output logic       dp );

logic [2:0] s;

logic [4:0] digit;

logic [19:0] clkdiv;

assign dp =1;

assign s = clkdiv[19:17];

always_comb

case(s)

0: digit = {{1'b0},x[3:0]};

1: digit = {{1'b0},x[7:4]};

2: digit = {{1'b0},x[11:8]};

3: digit = {{1'b1},x[15:12]};

4: digit = {{1'b0},x[19:16]};

5: digit = {{1'b0},x[23:20]};

6: digit = {{1'b0},x[27:24]};

7: digit = {{1'b0},x[31:28]};

default: digit = {{1'b0},x[4:0]};

endcase

always_comb

case(s)

0: an = 8'b1111_1110;

1: an = 8'b1111_1101;

2: an = 8'b1111_1011;

3: an = 8'b1111_0111;

4: an = 8'b1110_1111;

5: an = 8'b1101_1111;

6: an = 8'b1011_1111;

7: an = 8'b0111_1111;

default: an = 8'b1111_1111;

endcase

always @(posedge clk)

begin

clkdiv <= clkdiv + 1;

end

//实例化 7段数码管

Hex7Seg s7(.x(digit), .a2g(a2g));

Endmodule

module Hex7Seg(

input logic [4:0] x,

output logic [6:0] a2g    );

always_comb

case(x)

'h0: a2g = 7'b0000001;

'h1: a2g = 7'b1001111;

'h2: a2g = 7'b0010010;

'h3: a2g = 7'b0000110;

'h4: a2g = 7'b1001100;

'h5: a2g = 7'b0100100;

'h6: a2g = 7'b0100000;

'h7: a2g = 7'b0001111;

'h8: a2g = 7'b0000000;

'h9: a2g = 7'b0000100;

'ha: a2g = 7'b0000010;

'hb: a2g = 7'b1100000;

'hc: a2g = 7'b0110001;

'hd: a2g = 7'b1000010;

'he: a2g = 7'b0010000;

'hf: a2g = 7'b0111000;

'h10:a2g = 7'b1110110;

default: a2g = 7'b1111111;

endcase

endmodule

3.仿真

实验结果：

4.上板：（实验视频见文件夹）

【四】问题回顾与总结反思

问题回顾：

未区分寄存器写/读的下降沿/上升沿

刚开始誊写寄存器写入代码时，未将时钟触发条件设置为下降沿，对寄存器写/读进行区分。

总结反思：

本次实验主要考察了在单周期基础上对于流水线CPU原理的进一步认识，其中重点涉及了对流水寄存器和冲突处理的理解。由于流水线CPU是将指令分为多个阶段依次执行，不同阶段需要使用流水寄存器进行状态保存。对于数据/指令冲突，需要设置单独的hazard模块进行处理。在进行实验前期构思设计时，需遵照每个阶段依次厘清指令所经过的状态，并判断冲突类型。

通过本次实验，巩固了计算机组成的基础硬件知识与基本逻辑。相较单周期CPU、多周期CPU而言，流水线CPU因逻辑问题导致的错误进一步减少，调试时间也大大减少，说明对CPU的体系结构了解更深。由于粗心大意犯的错再次提醒在实验中细心与耐心的重要性。同时，在调试阶段，应该避免惯性思维，如认为寄存器触发条件一定为上升沿，导致未处理下降沿的错误一直没被发现。